All research complete. Here is the structured report.

=================================================================== MEDIAPIPE HANDS — WHAT GOOGLE THEMSELVES PUBLISHED AND DOCUMENTED

1. The MediaPipe Hands paper (Zhang et al. 2020, arXiv 2006.10214) — DOCUMENTED FACT

Authors: Fan Zhang, Valentin Bazarevsky, Andrey Vakunov, Andrei Tkachenka, George Sung, Chuo-Ling Chang, Matthias Grundmann (Google Research), June 2020.

Architecture (two-stage):

- Palm detector (not a hand detector): single-shot detector in the BlazeFace family, with an encoder-decoder (FPN-like) feature extractor "for a larger scene-context awareness even for small objects", focal loss to handle the huge anchor count, and square-only anchors (reduces anchors 3-5x). Palms are detected because they are rigid and near-square, avoiding the articulated-finger problem. Result: 95.7% average precision in palm detection (also quoted in the legacy docs).

- Hand landmark model: runs on the crop derived from the detection (or from the previous frame's landmarks) and regresses three heads simultaneously: (1) 21 landmarks with x, y and relative depth z, (2) a hand presence confidence scalar ("whether a reasonably aligned hand is indeed present"), (3) a handedness binary classification (left/right). So handedness is a learned classification head of the landmark CNN itself — a per-frame appearance decision, not a temporal one. No temporal model exists anywhere in the pipeline; every frame is classified independently.

z-coordinate training: relative depth is relative to the wrist and was learned only from synthetic images (rendered rigged 3D hand model). This is straight from the paper and is the documented reason screen-landmark z is the weakest channel.

Datasets: ~6K "in-the-wild" images (used alone for the palm detector), ~10K in-house gesture images (30 subjects), ~100K synthetic renders (24-bone rigged model, 36 blendshapes, 5 skin tones).

Accuracy (MSE normalized by palm size): real-world data only 16.1%, synthetic only 25.7%, combined 13.4%. Model variants: Light 1M params (MSE 11.83), Full 1.98M (10.05), Heavy 4.02M (9.817); latencies e.g. Pixel 3: 6.6 / 16.1 / 36.9 ms. The paper gives no handedness accuracy number — no Google source I found quantifies handedness accuracy.

Tracking across frames: palm detection runs on the first frame only; subsequent ROIs are derived from the previous frame's landmarks; the detector is re-triggered only "when the confidence [hand presence score] falls below a certain threshold". This design is unchanged in the Tasks API (verified in source, §6).

2. MediaPipe's own smoothing — DOCUMENTED FACT (verified in source code)

Key finding: MediaPipe applies NO smoothing to hand landmarks — not in the legacy Hands solution, not in the Tasks HandLandmarker. I fetched both graphs:

- mediapipe/modules/hand_landmark/hand_landmark_tracking_cpu.pbtxt (legacy): no LandmarksSmoothingCalculator, no filter of any kind.

- mediapipe/tasks/cc/vision/hand_landmarker/hand_landmarker_graph.cc (Tasks): likewise none. (Gesture Recognizer wraps this same graph, so it is unfiltered too.) Raw model output is what you get; any jitter reduction is the app's job. The Tasks docs also never mention smoothing.

The LandmarksSmoothingCalculator exists as infrastructure (mediapipe/calculators/util/landmarks_smoothing_calculator.proto) with three filters and these defaults:

- NoFilter (default behavior)

- VelocityFilter: window_size=5, velocity_scale=10.0

- OneEuroFilter: frequency=30.0 fps, min_cutoff=1.0, beta=0.0, derivate_cutoff=1.0, min_allowed_object_scale=1e-6, disable_value_scaling=false. Proto comments give Google's own tuning advice: tune min_cutoff first with beta=0 to kill jitter, then raise beta to kill lag. Value scaling normalizes by object size so filter cutoffs are scale-invariant.

Where Google actually USES it is Pose, and their production tuning there is instructive (pose_landmark_filtering.pbtxt):

- Screen landmarks: OneEuro min_cutoff=0.05, beta=80, derivate_cutoff=1.0

- Auxiliary ROI landmarks: OneEuro min_cutoff=0.01, beta=10

- World landmarks: OneEuro min_cutoff=0.1, beta=40, derivate_cutoff=1.0, disable_value_scaling=true — i.e., when Google smooths world (metric) landmarks they disable object-scale normalization and use a stiffer min_cutoff than for screen landmarks. Visibility is smoothed with a simple low-pass alpha=0.1. So the answer to "what do THEY use by default for hands" is: nothing; and their reference tuning for an equivalent problem is the pose OneEuro config above, applied to both screen and world coordinates with different parameters.

3. World-landmark scale / bone-length inconsistency — MIXED: documented mechanism + community-confirmed symptom

How world landmarks are actually made (documented, TensorFlow blog Nov 2021 + Sung et al. arXiv 2111.00038):

- Ground truth came from fitting the statistical GHUM hand model to their existing 2D annotations and recovering 3D coordinates — not from real depth capture.

- The paper explicitly states the assumption that "human hands have minor size variations reflected in hand shape", and for unknown cameras focal length is assumed = max(image width, height). In other words, world landmarks are pseudo-metric, pinned to a near-average GHUM hand; absolute scale is not measured from your hand.

- The world-coordinate origin is "an auxiliary keypoint formed as an average between the first knuckles of index, middle, ring and pinky fingers" (TF blog); the legacy docs phrase it as "the hand's approximate geometric center".

- Documented accuracy: mean 3D error 1.3 cm (Full) / 1.4 cm (Lite) on an ASL evaluation; the 2021 paper quotes ~1.5 cm mean prediction error. Against a ~9 cm palm, 1.3-1.5 cm mean error is entirely consistent with the 5-20% pose-dependent bone-length variation you measured — but Google nowhere documents "bone lengths vary with pose" as an explicit statement; that is my inference from their error numbers plus community evidence.

Community/GitHub evidence (reported, partly Google-acknowledged by triage but never fixed):

- Issue #5156 "HAND WORLD LANDMARKS collapse for the back of the hand" (v0.10.9, Feb 2024): palm/MCP world landmarks collapse specifically when the back of the hand faces the camera; reporter notes it re-occurs from earlier #3994; still open, "awaiting googler", no fix.

- Issue #3156: screen-landmark depth shrinkage and world-landmark width shrinkage, and behavior changed between versions 0.8.7 → 0.8.9 and between lite/full variants. Closed without documented resolution.

- Issue #5571 "Hand World Landmarks are not accurate" — 3D plots look wrong following official docs.

- Issue #742: screen-landmark z "not normalized, doesn't adhere to any coherent scale". Official doc statement on screen z: "z represents the landmark depth with the depth at the wrist being the origin... magnitude of z uses roughly the same scale as x" — "roughly" is the only precision Google commits to, and z was trained on synthetic data only (paper).

Bottom line for your pipeline: it is documented fact that world landmarks are a GHUM-average-hand reconstruction with ~1.3-1.5 cm mean error and an orientation-sensitive failure mode (back-of-hand collapse, open issue); pose-dependent bone-length variance of the magnitude you see is expected and not something Google claims to guarantee against.

4. Handedness reliability — DOCUMENTED convention + community-reported instability

Documented convention (legacy Hands docs, verbatim): "handedness is determined assuming the input image is mirrored, i.e., taken with a front-facing/selfie camera with images flipped horizontally. If it is not the case, please swap the handedness output in the application." So on a non-flipped webcam feed the labels are intentionally inverted — this is the documented root of your production mirroring bug class.

Community-reported instability (GitHub, none with a documented fix):

- #3047: "Hand Left/Right Classification is confused (i.e. wrong often)" — notably when the hand is near the frame edge or partially overlapping the other hand.

- #4785: wrong left/right classification when only one hand is in frame.

- #4724: handedness flipped in Gesture Recognizer via webcam (Tasks-era report).

- #4610: multi_handedness gives the wrong index in classification. I found no Google statement quantifying handedness accuracy or addressing back-of-hand/rotation flipping specifically; the mechanism (per-frame CNN head, §1, no temporal consistency) makes flip-during-rotation an expected behavior — that link is my inference, but the per-frame architecture is documented fact.

5. Newer Google work — DOCUMENTED

- Sung et al. 2021, "On-device Real-time Hand Gesture Recognition" (arXiv 2111.00038) — the most important follow-up for you. Documents an improved hand landmark model (trained with ASL/complex poses: 71.3 mAP vs 66.5 mAP for original MediaPipe Hands), the introduction of metric world landmarks via GHUM fitting (details in §3), an improved rotation-normalization for the ROI (two-vector scheme: middle-MCP→wrist plus index-MCP→pinky-MCP, replacing the 2D wrist→middle-finger vector, "stable when hand approaches frontal view"), and gesture classifiers (heuristic vs a 3x50-neuron FC net, 87.9% avg recall at 1% FPR over 6 static gestures).

- MediaPipe Holistic (Google AI Blog, Dec 2020): no dedicated hand paper; derives hand ROIs from pose keypoints plus lightweight re-crop models (~10% of model inference cost) because pose-derived ROIs are too coarse; up to 8 models/frame. Relevant idea: using an upstream body/arm signal to stabilize the hand crop. Now also exists as the Tasks "Holistic Landmarker".

- Tasks-era hand landmark bundle ("HandLandmarker (full)", input 192x192 + 224x224, float16, Pixel 6 CPU 17.12 ms / GPU 12.27 ms) is documented as trained on "~30K real-world images" plus synthetic — versus ~16K real in the 2020 paper — i.e., the shipped model is the improved 2021-generation model, not the 2020 one. Evidence that model behavior changed across releases: issue #3156 (0.8.7 vs 0.8.9 depth differences). I found no announced hand-model update after the Tasks launch (2023) through today.

- HOnnotate/HandNeRF are NOT Google publications (my note for scoping; no Google temporal-hand-tracking paper found — the pipeline's only temporal component remains the ROI loopback).

6. Multi-hand identity across frames — DOCUMENTED FACT (from source + docs)

- Tasks HandLandmarker graph (verified in hand_landmarker_graph.cc): a PreviousLoopbackCalculator feeds the previous frame's hand rects back; a NormalizedRectVectorHasMinSizeCalculator gates the palm detector — it is skipped while the number of tracked hands >= num_hands ("If the tracking is lost or the detected hands are less than configured max number hands, HandDetectorGraph would be triggered"). New detections are merged with tracked rects by HandAssociationCalculator, whose min_similarity_threshold is set from min_tracking_confidence — and the official docs define min_tracking_confidence in the Tasks API as "the bounding box IoU threshold between hands in the current frame and the last frame" (default 0.5). So in Tasks, min_tracking_confidence is an IoU/association threshold, NOT a model confidence.

- Legacy solution semantics differ: there min_tracking_confidence is the "minimum confidence value from the landmark-tracking model for the hand landmarks to be considered tracked successfully, or otherwise hand detection will be invoked automatically on the next input image" — a presence-score threshold. The legacy graph uses a fixed AssociationNormRectCalculator with hardcoded min_similarity_threshold 0.5 for deduplication.

- Neither API assigns or documents a persistent hand ID. Output vector order is not guaranteed stable across frames; identity continuity is only an emergent side effect of ROI loopback and IoU association, and it breaks exactly when the detector re-fires (tracking loss, hand entering/leaving). This matches your "Object Jump Correction" root cause: MediaPipe provides no identity mechanism to preserve, so per-app identity matching (spatial proximity/appearance) is the standard community remedy. Documented fact: no ID in the output types; community consensus: order is unreliable.

=================================================================== SOURCES

Papers / blogs (Google):

- https://arxiv.org/abs/2006.10214 (MediaPipe Hands paper; HTML at https://ar5iv.labs.arxiv.org/html/2006.10214)

- https://arxiv.org/abs/2111.00038 (On-device Real-time Hand Gesture Recognition, Sung et al. 2021)

- https://blog.tensorflow.org/2021/11/3D-handpose.html (3D Hand Pose with MediaPipe — GHUM world landmarks, 1.3/1.4 cm errors)

- https://research.google/blog/mediapipe-holistic-simultaneous-face-hand-and-pose-prediction-on-device/ (Holistic, Dec 2020)

Official docs:

- https://chuoling.github.io/mediapipe/solutions/hands.html (legacy Hands docs: mirroring note, z definition, world landmark definition, option semantics, 95.7% AP)

- https://developers.google.com/edge/mediapipe/solutions/vision/hand_landmarker (Tasks docs: options/defaults, IoU semantics of min_tracking_confidence, model bundle, benchmarks)

- https://developers.google.com/edge/mediapipe/solutions/vision/holistic_landmarker

Source code (verified directly):

- https://github.com/google-ai-edge/mediapipe/blob/master/mediapipe/calculators/util/landmarks_smoothing_calculator.proto (filter defaults)

- https://github.com/google-ai-edge/mediapipe/blob/master/mediapipe/modules/hand_landmark/hand_landmark_tracking_cpu.pbtxt (legacy graph — no smoothing)

- https://github.com/google-ai-edge/mediapipe/blob/master/mediapipe/tasks/cc/vision/hand_landmarker/hand_landmarker_graph.cc (Tasks graph — no smoothing, association logic)

- https://github.com/google-ai-edge/mediapipe/blob/master/mediapipe/modules/pose_landmark/pose_landmark_filtering.pbtxt (Google's own OneEuro tuning)

GitHub issues (community reports):

- https://github.com/google-ai-edge/mediapipe/issues/5156 (world landmarks collapse, back of hand — open)

- https://github.com/google/mediapipe/issues/3156 (depth/width shrinkage, version-dependent)

- https://github.com/google-ai-edge/mediapipe/issues/5571 (world landmarks inaccurate)

- https://github.com/google/mediapipe/issues/742 (z scale incoherent)

- https://github.com/google/mediapipe/issues/3047, /4785, /4724, /4610 (handedness wrong/flipped)

- https://github.com/google/mediapipe/issues/502 (per-hand identity across frames)

PowerShell Run jump-provenance audit over recorded corpus

IN

cd Local_pc\Movement_with_hand_detection; .venv\Scripts\python.exe analysis\audit_jump_provenance.py

OUT

cd : Cannot find path 'C:\Users\sugit\Documents\_scripts_persos\_Persos\vision_pipeline_python\Hand_detection\Local_pc\

Movement_with_hand_detection\Local_pc\Movement_with_hand_detection' because it does not exist.

At line:1 char:302

+ ... ext' } catch {} } }; cd Local_pc\Movement_with_hand_detection; .venv\ ...

+                          ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

+ CategoryInfo          : ObjectNotFound: (C:\Users\sugit\..._hand_detection:String) [Set-Location], ItemNotFoundE

xception

+ FullyQualifiedErrorId : PathNotFound,Microsoft.PowerShell.Commands.SetLocationCommand

--- V0 raw label, no guards (published method) ---

frames 22345   >30 1533   >60 730   >60 at obs>=0.60: 597 (81.8%)

worst sessions (>60): free_manipulation=280, palm_back_s2_slow=129, palm_back_s4_fast=90, palm_back_s3_medium=81, pitch_sweep_slow=41, pitch_sweep_fast=34, palm_back_s1_very_slow=30, two_hand_overlap=24

--- V1 + dup-frame skip + continuity guard ---

frames 22137   >30 1194   >60 482   >60 at obs>=0.60: 352 (73.0%)

worst sessions (>60): free_manipulation=138, palm_back_s2_slow=126, palm_back_s4_fast=65, palm_back_s3_medium=63, pitch_sweep_slow=41, palm_back_s1_very_slow=30, pitch_sweep_fast=17, two_hand_crossing=1

--- V2 DR-1 identity-corrected + guards ---

frames 22345   >30 1350   >60 572   >60 at obs>=0.60: 440 (76.9%)

worst sessions (>60): free_manipulation=201, palm_back_s2_slow=126, palm_back_s4_fast=68, palm_back_s3_medium=60, pitch_sweep_slow=41, palm_back_s1_very_slow=30, pitch_sweep_fast=25, two_hand_crossing=15

OPEN-LOOP 1-frame model error (all triples, n=22048): median 4.50  mean 12.55  p90 28.37  p99 144.90  >15deg 18.4%  >25deg 11.4%

OPEN-LOOP 1-frame model error (seed delta < 30 deg, n=20734): median 4.15  mean 8.64  p90 18.59  p99 82.34  >15deg 13.3%  >25deg 6.4%

chi2_probe's published figure was 59.9% rejected at >25 deg -- that number

included closed-loop cascade rejections, not one-frame prediction error.

Web Fetchhttps://ar5iv.labs.arxiv.org/html/1903.07933

Fetched from https://ar5iv.labs.arxiv.org/html/1903.07933

All research complete. Here is the structured report.

=================================================================== LITERATURE REPORT: STABILIZING MONOCULAR RGB HAND TRACKING (2023–2026) Context: MediaPipe Hands, single webcam ~24fps, CPU, 3D object manipulation

TOPIC 1 — SOTA monocular RGB 3D hand pose/mesh estimators

Documented landscape:

- HaMeR (CVPR 2024, Pavlakos et al.): ViT-Huge transformer regressing MANO; still the accuracy reference. Purely per-frame; no temporal model. Self-occlusion robustness comes from data scale, not from any occlusion-specific mechanism.

- WiLoR (arXiv 2409.12259, CVPR 2025): two parts — a real-time fully-convolutional multi-hand DETECTOR (130–175 fps, 25 MB model, 45x faster and 32x smaller than prior SOTA detector) plus a high-fidelity transformer reconstructor. The detector is lightweight; the reconstructor is not. Per-frame only.

- Hamba (NeurIPS 2024, arXiv 2407.09646): graph-guided bi-scanning Mamba (state-space model); PA-MPVPE 5.3 mm, F@15mm 0.992 on FreiHAND. Per-frame only.

- HandOS (CVPR 2025, arXiv 2412.01537): one-stage — detection + 2D pose + 3D mesh in a single frozen-detector framework, removing cumulative errors of the detect→classify→regress cascade; 5.0 PA-MPJPE FreiHAND, 64.6% PCK@0.05 HInt-Ego4D. Per-frame only.

- Dyn-HaMR (CVPR 2025 Highlight, arXiv 2412.12861): offline multi-stage OPTIMIZATION pipeline (SLAM + interacting-hand generative prior + hierarchical init) for 4D global hand motion from moving cameras. Explicitly not real-time.

- HaWoR (CVPR 2025 Highlight, arXiv 2501.02973): world-space hand motion from egocentric video; includes a "motion infiller" network that generatively completes missing/out-of-frustum frames — directly relevant concept for your bad-frame problem, but built for offline egocentric video.

- OmniHands (arXiv 2405.20330): "robust 4D hand mesh recovery via a versatile transformer" — temporal transformer variant of the HaMeR family for video.

- 2026 items: Fast-HaMeR (arXiv 2603.16444, Mar 2026): knowledge-distilled HaMeR to MobileNet/MobileViT/ConvNeXt/ResNet backbones — 35% of original size, 1.5x faster, only 0.4 mm accuracy loss. HandFlow (arXiv 2607.11221, 2026): fully generative 4D hand recovery with flow matching (note: name collision with the 2022 HandFlow ambiguity paper below). DF-Mamba (arXiv 2512.02727): deformable state-space modeling for hands in interaction.

Temporal stability: a recurring documented statement (e.g., in the OmniHands and HandFlow-2026 papers) is that HaMeR/WiLoR/Hamba "process each image independently and produce temporally inconsistent outputs on video." The field's fixes are (a) video transformers (OmniHands, Deformer), (b) offline optimization with motion priors (Dyn-HaMR, HMP), (c) post-hoc refinement (SmoothNet, Topic 2). No per-frame SOTA model solves your jump problem by itself.

CPU feasibility (documented + inference):

- Documented: none of HaMeR/Hamba/HandOS/Dyn-HaMR are CPU real-time; they are GPU transformer/Mamba models. MobRecon (CVPR 2022, arXiv 2112.02753) remains the reference for genuinely light mesh recovery: 67–83 fps on an Apple A14 mobile CPU, with an explicit temporal-coherence design goal.

- Inference: on a desktop x86 CPU in Python, Fast-HaMeR-class distilled models are still likely several hundred ms/frame; MobRecon-class (~million-param CNN + spiral conv) is the only mesh-model family plausibly near your 24 fps budget. Realistic use of the big models for you: OFFLINE oracle to label/validate your recorded clips (e.g., run WiLoR/HaMeR on E: recordings to quantify MediaPipe error per pose class), not as the live estimator.

TOPIC 2 — Temporal refinement / what the field actually uses for glitches

- SmoothNet (ECCV 2022, arXiv 2112.13715): the canonical plug-and-play answer. Key documented insight matching your pain point (a): pose-estimator error is "highly unbalanced" across frames — most frames are fine, and failures are LARGE deviations over short consecutive runs, not white noise. SmoothNet is a temporal-ONLY fully-connected network per joint over a sliding window (they use long windows, e.g. 32–64 frames), modeling position+velocity+acceleration; it both smooths jitter and fixes the large glitch frames "as a side-effect." Strong documented transferability across estimators/modalities. Feasibility: it is a tiny MLP — CPU real-time is plausible (inference), but the sliding window adds latency unless run in causal mode with a window ending at the current frame (accuracy drops somewhat in their causal ablation — check paper Table 9 before committing).

- Simple filtering: the One Euro filter (CHI 2012) remains the documented default in interactive systems (used by Monado's open-source OpenXR hand tracking; MediaPipe itself ships One-Euro/velocity "landmarks smoothing calculator" in Holistic but NOT exposed in the Hands solution — GitHub issue #3354). One Euro is a jitter/lag tradeoff tool only — it does not reject glitches; a large bad-frame excursion passes through (documented behavior: adaptive cutoff opens up at high velocity, so a glitch that looks like fast motion is followed, not suppressed).

- Learned motion priors: HMP (WACV 2024, arXiv 2312.16737) — generative hand motion prior trained on AMASS mocap; video fitting via latent optimization; documented to significantly improve accuracy under partial/heavy occlusion on DexYCB/HO3D. Offline (optimization per sequence). 2026 direction: video diffusion / flow-matching priors (arXiv 2606.30308, 2607.11221) — also offline.

- Temporal transformers: Deformer (ICCV 2023, arXiv 2303.04991) — spatial+temporal transformer with dynamic fusion; documented +10% accuracy overall and +14% under occlusion vs per-frame baselines. HTT (arXiv 2209.09484) similar. GPU-class.

- Field practice for glitch rejection (synthesis, documented in robotics literature, e.g. arXiv 2605.02708 "Temporally Consistent Object 6D Pose Estimation"): probabilistic smoothing/temporal recursion that "rejects short-lived outliers and enforces physically plausible temporal continuity" — i.e., a filter with an explicit outlier gate (Mahalanobis/innovation gating in a Kalman-style loop), not raw per-frame confidence. Per-frame confidence scores from estimators (MediaPipe's handedness/presence scores included) are documented in multiple threads to be poorly calibrated for exactly the well-conditioned-pose failures you see; papers instead use consistency measures: reprojection error, bone-length deviation, velocity/acceleration limits, left-right or multi-crop agreement.

- Direct recommendation for your pain point (a) (inference, but well grounded): your measured signal — bone-length variation — is itself the standard glitch detector in this literature. An innovation-gated filter (reject/hold when bone-length deviation or angular velocity exceeds a gate, One-Euro or Kalman otherwise) is the documented lightweight pattern; SmoothNet-causal is the documented learned upgrade if you can tolerate a small window.

TOPIC 3 — Orientation/depth-sign ambiguity, degenerate views

- HandFlow (VMV 2022, MPI, Best Paper HM): the key citable paper on exactly this. Models the POSTERIOR over hand poses given one image with a conditional normalizing flow; documents that single-pose methods miss valid alternative reconstructions that project identically (depth-sign / bas-relief family), that standard metrics can't even evaluate this, and releases MultiHands (multiple plausible GT annotations per image). Core documented recommendation: under severe ambiguity, model the distribution, don't regress one pose.

- Multi-hypothesis / probabilistic pose is an established line for the same reason in body pose (e.g., arXiv 2411.13026 documents that ambiguous 2D evidence yields a MULTIMODAL depth distribution — multiple peaks, "greater variance in z than x,y"); diffusion/flow-matching probabilistic pose continues this in 2025–26 (arXiv 2602.03126, 2602.05755).

- Anatomical/biomechanical constraints: Spurr et al. ECCV 2020 (arXiv 2003.09282) is the standard citation — joint-angle-limit + bone-structure losses cut DEPTH error by 50% on FreiHAND vs 15% without constraints, i.e., anatomy resolves a large share of the depth/orientation ambiguity. PyTorch reimplementation exists (MengHao666/Hand-BMC-pytorch) and the constraint set (valid flexion/abduction ranges, planar finger articulation) can be applied at inference as a pose validity check, not just as a training loss (that use is inference on my part, but the constraint definitions are published).

- Rotation through degenerate views specifically: no paper found that names "palm edge-on to camera" as a studied case for RGB hand tracking (absence of evidence, stated as such). The closest documented practices: (i) UmeTrack/MEgATrack solve it with MULTI-VIEW fusion — Meta's stated reason for multi-camera rigs is exactly degenerate/occluded single views; (ii) HaWoR/Dyn-HaMR treat bad segments as MISSING DATA and generatively infill from a motion prior — i.e., the SOTA answer to a degenerate window is "don't trust the estimator there; bridge it"; (iii) on the filtering side, quaternion-state EKF/UKF on SO(3) (Kraft UKF, quaternion EKF — IEEE 1643403; kinematic-chain quaternion EKF PMC5108752) is the documented machinery for smoothing orientation with proper manifold handling, gimbal-lock-free, including adaptive measurement-noise inflation when input reliability drops — the direct analogue of down-weighting MediaPipe during your pitch-plane crossing. These are IMU papers; applying them to vision-landmark-derived quaternions is standard practice but their accuracy numbers don't transfer (inference).

- Practical synthesis for pain point (b): the literature offers no per-frame cure for edge-on views from one RGB camera — it is a genuinely ill-posed configuration (HandFlow's thesis). Documented coping strategies are: widen measurement covariance / gate out measurements during the degenerate window and coast on a constant-angular-velocity or learned motion prior (Quest-style prediction, HaWoR-style infill), and use anatomical validity + bone-length consistency to decide when the window has ended.

TOPIC 4 — MANO fitting as a stabilizer

- Documented mechanism: MANO gives bone lengths fixed by shape parameters (β) by construction; fitting β once (or per-session) and optimizing only pose per frame guarantees pose-consistent bone lengths — the property MediaPipe world landmarks lack.

- "3D Hand Shape and Pose Estimation based on 2D Hand Keypoints" (ACM PETRA 2023): fits MANO to 2D keypoints with an objective = 2D reprojection + BONE-LENGTH PRESERVATION + regularization — direct documented precedent for your use case.

- Constrained-IK hand tracking (Aristidou 2010/2018) documents keeping bone lengths constant over time via IK for continuous data flow; V-Hands (arXiv 2409.13347) and multi-view pipelines document the recipe: average per-subject bone lengths across a sequence, then adjust skeleton, then learning-based IK for rotations.

- "Monocular Biomechanical Tracking of Fingers with Inverse Kinematics to Foundation Models" (arXiv 2605.09258, 2026): recent, directly on-topic — applies biomechanical IK on top of foundation-model (HaMeR-class) outputs for anatomically consistent finger tracking. I could not extract its runtime numbers from search results.

- Does literature show fitting stabilizes orientation vs raw landmarks? Documented: HMP and Dyn-HaMR (both optimization against MANO + priors) show large gains under occlusion vs per-frame landmark/mesh regressors. Not found: a paper that isolates "MANO fit vs raw MediaPipe landmarks" head-to-head with bone-length variance numbers — your 1%→25% measurement is, as far as I can tell, not published anywhere; the closest is the MediaPipe GitHub issue record (#5571 "Hand World Landmarks are not accurate", #3156 depth/width shrinkage, #2845 depth regression between model versions) confirming the phenomenon qualitatively.

- Cost (inference from documented structure): per-frame MANO pose fitting is a ~26–45-DoF nonlinear least squares; warm-started from the previous frame it typically converges in a few Gauss-Newton/LM iterations. In NumPy/PyTorch-CPU with an analytic or autodiff Jacobian this is plausibly a few ms — feasible at 24 fps — but no paper documents Python-CPU timings; budget a prototype. A cheaper documented variant: skip full MANO, fix YOUR OWN skeleton (median bone lengths from calibration) and run constrained IK on MediaPipe's 2D landmarks + depth-lifted root — this is the V-Hands/Aristidou pattern and directly kills pain point (c).

TOPIC 5 — Metric depth / Z from monocular RGB

- Geometric anchor method (documented, multiple sources): D = f·H/h (similar triangles) with a known physical anchor. For hands specifically, arXiv 2504.01888 (rehab exoskeleton gesture control) uses PALM WIDTH between hand keypoints as the real-world anchor with camera focal length — the palm-width baseline is preferred over finger-based spans because it is nearly pose-invariant (finger spans foreshorten). Accuracy claims in that class of work are in the few-cm range at ~0.5–1 m, but I did not find a rigorous cm-accuracy benchmark for hand-anchored webcam depth — treat "~cm" as plausible-not-proven.

- RootNet (ICCV 2019, arXiv 1907.11346): the canonical learned version — estimates root depth from the ratio of real-world reference area to image crop area, corrected by an image-feature-driven factor; the standard citation for absolute-root recovery in top-down pipelines.

- ScaleHP (arXiv 2606.25619, 2026): current SOTA framing — uses intrinsic PROPORTIONAL RELATIONSHIPS AMONG HAND BONES as anthropometric priors plus a scale token and perspective-constrained least-squares to recover camera-space metric coordinates, explicitly avoiding "fragile depth estimation modules"; 35.8 mm CS-MPJPE on FreiHAND (camera-space error — indicates absolute hand-position error of ~3.5 cm is state of the art per-frame, a useful calibration for your expectations).

- Camera-distance-aware hand work: "Camera distance helps 3D hand pose estimated from a single RGB image" (ScienceDirect 2023) documents that depth-dependent scale ambiguity is governed by joint-to-lens distance and that supplying camera distance improves pose.

- General monocular metric depth (Depth Pro, Metric3D family): documented to be sensitive to camera intrinsics and not real-time CPU; not the right tool for a single hand (inference).

- For your planned Z-gesture (inference, grounded): the documented consensus is that RELATIVE depth ratio (hand-scale ratio vs a calibrated rest pose) is far more reliable than absolute metric depth — RootNet/ScaleHP-style absolute error is ~3–4 cm/frame, while scale-ratio control only needs temporal consistency of one anchor length. Use palm width (MCP2–MCP5 or wrist–MCP span) in the 2D image + One-Euro on the ratio; avoid finger-dependent lengths; gate during the pitch-degenerate window since the palm-width pixel span collapses edge-on.

TOPIC 6 — What shipping AR/VR systems do

- Meta Quest: MEgATrack (SIGGRAPH 2020) — 4 monochrome fisheye cameras; detection-by-tracking (keypoint network CONSUMES PREVIOUS-FRAME POSE to produce "spatially and temporally consistent," explicitly "low-jitter" output); model-fitting of a hand skeleton to network keypoints. UmeTrack (SIGGRAPH Asia 2022) — end-to-end differentiable multi-view + TEMPORAL FUSION network predicting absolute world-space pose. Product releases are documented but architecture-sparse: Hands 2.1 (2022–23) added "a new neural network to improve prediction accuracy — less overshooting, smoother poses, especially fast motion" and cut reacquisition time; Hands 2.2 (2023) claims up to 40% latency reduction typical, 75% during fast movement, via prediction/extrapolation ("almost as responsive as controllers"); Hands 2.4 improved Fast Motion Mode further. Takeaway (documented): Meta's answer to fast motion is FORWARD PREDICTION with a learned model, plus multi-view for occlusion/degeneracy; jitter is handled inside the temporal network, not by post-hoc filtering.

- Apple Vision Pro: no published architecture paper. Documented via visionOS 2 release: hand anchors served at 90 Hz with an optional PREDICTION mode giving "extrapolated hand poses for display time — lower effective latency at some accuracy cost" (uploadvr; Apple dev docs). Independent measurement (PMC12390506, 2025 MR-boxing paper): motion-to-feedback latency 18.3±2.1 ms, effective update ~60 Hz. RoadToVR published an AVP-vs-Quest3 photon-latency comparison (Quest 3 slightly lower with FMM).

- Ultraleap: Gemini→Hyperion (2024). Documented: IR stereo cameras, "more than a decade of machine learning," multiple swappable per-use-case tracking models, mm-level microgesture claims, tracking-while-holding-objects. Historic Leap architecture was model-based fitting to stereo IR; current docs describe it as ML model zoo — your note "model-based fitting" is accurate for classic Leap but Hyperion is presented as learned models (documented shift).

- HoloLens 2: articulated hand tracking runs on a dedicated 45 fps active NEAR-DEPTH time-of-flight camera (AHAT) + HPU, not RGB — 25 joints ≤1 m. Independent evaluations (Virtual Worlds 2025; ScienceDirect 2025 crowdsourced eval) document persistent failures with overlapping/occluded fingers even with depth hardware — useful calibration: occlusion is hard even with ToF.

- Cross-cutting documented pattern: every shipping system uses (1) temporal state fed into the network (detection-by-tracking), (2) forward prediction/extrapolation to hide latency, (3) extra sensing (multi-view, IR, ToF) precisely for the degenerate/occluded cases a single RGB view cannot resolve. None relies on per-frame independent estimation plus post-filtering — which is what a raw MediaPipe pipeline is (last clause is inference/synthesis).

ACTIONABLE SYNTHESIS FOR YOUR PIPELINE (inference, grounded in the above)

- Pain (a) big jumps in good poses: innovation-gated quaternion filter (UKF/EKF on SO(3) or gated One-Euro on quats) using bone-length deviation + angular-velocity gates as the outlier trigger; SmoothNet-causal as the learned upgrade path (tiny MLP, CPU-viable).

- Pain (b) pitch-plane crossing: treat as detectable degenerate window (bone-length variance + palm-width pixel collapse are your detectors); inflate measurement noise/coast on constant angular velocity through it (Quest-style prediction, HaWoR-style bridging). No published per-frame fix exists for single RGB.

- Pain (c) bone-length inconsistency: calibrate a fixed personal skeleton, then constrained IK / lightweight MANO-pose-only fit per frame (bone lengths fixed by construction; documented precedent PETRA 2023, V-Hands, arXiv 2605.09258). This also gives you a cleaner quaternion source than raw landmarks.

- Z-depth: palm-width scale-ratio control (relative, calibrated), not absolute metric depth; absolute SOTA is only ~3.5 cm (ScaleHP) anyway; gate during edge-on windows.

- Offline oracle: run WiLoR or HaMeR (GPU/collab, offline) on your E: recordings to get pseudo-ground-truth for tuning gates/filters — the standard evaluation trick in this literature.